# Lab 09 – Worksheet

| Name:Syed Asghar Abbas Zaidi | ID:07201 | Section:T6 |
| --- | --- | --- |

## Task a. 4:1 MUX

Complete the truth table provided in Table 9.2. As an example, when the Select line input is S1:0=00, output will be the binary number stored in A3:0

Table 9.2

| S1 | S0 | A3:0 | B3:0 | C3:0 | D3:0 | Y3:0 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0111 | X | X | X | 0111(7) |
| 0 | 1 | X | 0010 | X | X | 0010(2) |
| 1 | 0 | X | X | 0000 | X | 0000(0) |
| 1 | 1 | X | X | X | 0001 | 0001(1) |

Provide appropriately commented code for designed module

module mux_4_1( input [3:0] A, input [3:0] B, input [3:0] C, input [3:0] D, input [1:0] S, output [3:0] O ); assign O[3:0] = S[1] ? (S[0] ? D : C) : ( S[0] ? B : A ); endmodule

Write a testbench to thoroughly test designed mux_4_1 module.

`timescale 1ns / 1ps module Multiplex_Test(); reg [3:0] A; reg [3:0] B; reg [3:0] C; reg [3:0] D; reg [1:0] S; wire [3:0] O; mux_4_1 module_u_test (A,B,C,D,S,O); initial begin //is there a way to not write A,B,C,D again and again? #100 A = 3'b111; B = 3'b010; C = 3'b000; D = 3'b001; S = 2'b00; #100 A = 3'b111; B = 3'b010; C = 3'b000; D = 3'b001; S = 2'b01; #100 A = 3'b111; B = 3'b010; C = 3'b000; D = 3'b001; S = 2'b10; #100 A = 3'b111; B = 3'b010; C = 3'b000; D = 3'b001; S = 2'b11; end endmodule

Attach screenshot of Simulation output- make sure to scale properly for visibility of all case.

## Task b. 1:4 DEMUX

Complete the truth table provided in Table 9.4. As an example, when the Select line input is S1:0=00, output EnA will be 0 and the rest of the outputs will remain high.

Table 9.4

| S1 | S0 | En | EnA | EnB | EnC | EnD |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 0 | 1 |
| 1 | 1 | 0 | 1 | 1 | 1 | 0 |
| X | X | 1 | 1 | 1 | 1 | 1 |

Provide appropriately commented code for designed module

module demux_1_4( input En, input [1:0] S, output EnA, output EnB, output EnC, output EnD ); assign EnA = En ? (1'b1) : (S[1] ? (S[0] ? 1'b1 : 1'b1) : ( S[0] ? 1'b1 : 1'b0 )); assign EnB = En ? (1'b1) : (S[1] ? (S[0] ? 1'b1 : 1'b1) : ( S[0] ? 1'b0 : 1'b1 )); assign EnC = En ? (1'b1) : (S[1] ? (S[0] ? 1'b1 : 1'b0) : ( S[0] ? 1'b1 : 1'b1 )); assign EnD = En ? (1'b1) : (S[1] ? (S[0] ? 1'b0 : 1'b1) : ( S[0] ? 1'b1 : 1'b1 )); endmodule

Add your testbench here

`timescale 1ns / 1ps module demux_4_Simulation(); reg En; reg [1:0] S; wire EnA; wire EnB; wire EnC; wire EnD; demux_1_4 module_u_test (En,S,EnA,EnB,EnC,EnD); initial begin //is there a way to not write A,B,C,D again and again? #100 En = 1'b0; S = 2'b00; #100 En = 1'b0; S = 2'b01; #100 En = 1'b0; S = 2'b10; #100 En = 1'b0; S = 2'b11; #100 En = 1'b1; S = 2'b00; #100 En = 1'b1; S = 2'b01; #100 En = 1'b1; S = 2'b10; #100 En = 1'b1; S = 2'b11; end endmodule

Attach screenshot of waveform results here

## Exercise

Write a Verilog module topLevelModule that combines MUX (mux_4_1) and DeMUX (demux_1_4) modules created in this lab and binary-hexadecimal decoder module already created in Lab 6, according to the block diagram of Figure 9.2 Top Level Module. This module’s inputs includes four 4-bit data lines (i.e., A[3:0], B[3:0], C[3:0], D[3:0]), 2-bit select line (i.e. S[1:0]) to select from the data for binary-hexadecimal decoder, and a 1-bit active low enable signal (i.e. En). The modules output includes one 7-bit data line for seven segments (i.e., Y [6:0]) and four 1-bit Enable signals (i.e., EnA, EnB, EnC, EnD).

Provide appropriately commented code for your design module

`timescale 1ns / 1ps module topLevelModule( input [3:0] A, input [3:0] B, input [3:0] C, input [3:0] D, input [1:0] S, input En, output [6:0] Y, output EnA, output EnB, output EnC, output EnD ); wire [3:0] O; mux_4_1 Mux(A,B,C,D,S,O); demux_1_4 DeMux(En,S,EnA,EnB,EnC,EnD); SevenSegment SS (O,Y); endmodule module mux_4_1( input [3:0] A, input [3:0] B, input [3:0] C, input [3:0] D, input [1:0] S, output [3:0] O ); assign O[3:0] = S[1] ? (S[0] ? D : C) : ( S[0] ? B : A ); endmodule module demux_1_4( input En, input [1:0] S, output EnA, output EnB, output EnC, output EnD ); assign EnA = En ? (1'b1) : (S[1] ? (S[0] ? 1'b1 : 1'b1) : ( S[0] ? 1'b1 : 1'b0 )); assign EnB = En ? (1'b1) : (S[1] ? (S[0] ? 1'b1 : 1'b1) : ( S[0] ? 1'b0 : 1'b1 )); assign EnC = En ? (1'b1) : (S[1] ? (S[0] ? 1'b1 : 1'b0) : ( S[0] ? 1'b1 : 1'b1 )); assign EnD = En ? (1'b1) : (S[1] ? (S[0] ? 1'b0 : 1'b1) : ( S[0] ? 1'b1 : 1'b1 )); endmodule module SevenSegment( input [3:0] D, output [6:0] S); assign S[0] = ((~D[3])&(~D[2])&(~D[1])&D[0]) ¦ ((~D[3])&(D[2])&(~D[1])&(~D[0])) ¦ ((D[3])&(~D[2])&(D[1])&D[0]) ¦ ((D[3])&(D[2])&(D[1])&D[0]); assign S[1] = ((D[2])&(D[1])&(~D[0])) ¦ ((D[3])&(D[1])&(D[0])) ¦ ((D[3])&(D[2])&(~D[0])) ¦ ((~D[3])&(D[2])&(~D[1])&D[0]); assign S[2] = ((D[3])&(D[2])&(~D[0])) ¦ ((D[3])&(D[2])&(D[1])) ¦ ((~D[3])&(~D[2])&(D[1])&(~D[0])); assign S[3] = ((D[2])&(D[1])&D[0]) ¦ ((~D[3])&(~D[2])&(~D[1])&(D[0])) ¦ ((~D[3])&(D[2])&(~D[1])&(~D[0])) ¦ ((D[3])&(~D[2])&(D[1])&(~D[0])); assign S[4] = ((~D[3])&D[0]) ¦ ((~D[2])&(~D[1])&(D[0])) ¦ ((~D[3])&(D[2])&(~D[1])); assign S[5] = ((~D[3])&(~D[2])&D[0]) ¦ ((~D[3])&(~D[2])&(D[1])) ¦ ((~D[3])&(D[1])&D[0]) ¦ ((D[3])&(D[2])&(~D[1])&D[0]); assign S[6] = ((~D[3])&(~D[2])&(~D[1])) ¦ ((~D[3])&(D[2])&(D[1])&(D[0])) ¦ ((D[3])&(D[2])&(~D[1])&(~D[0])); endmodule

Add your testbench here

`timescale 1ns / 1ps module topLevelModuleSimulation(); reg [3:0] A; reg [3:0] B; reg [3:0] C; reg [3:0] D; reg [1:0] S; reg En; wire [6:0] Y; wire EnA; wire EnB; wire EnC; wire EnD; topLevelModule module_u_test (A,B,C,D,S,En,Y,EnA,EnB,EnC,EnD); initial begin //is there a way to not write A,B,C,D again and again? #100 A = 3'b111; B = 3'b010; C = 3'b000; D = 3'b001; S = 2'b00; En = 1'b00; #100 A = 3'b111; B = 3'b010; C = 3'b000; D = 3'b001; S = 2'b01; En = 1'b00; #100 A = 3'b111; B = 3'b010; C = 3'b000; D = 3'b001; S = 2'b10; En = 1'b00; #100 A = 3'b111; B = 3'b010; C = 3'b000; D = 3'b001; S = 2'b11; En = 1'b0; #100 A = 3'b111; B = 3'b010; C = 3'b000; D = 3'b001; S = 2'b00; En = 1'b1; end endmodule

Attach screenshot of waveform results here

## Assessment Rubrics

Marks Distribution:

|  |  | LR2 Code | LR5 Results | LR7 Viva (Performance) | LR9 Report |
| --- | --- | --- | --- | --- | --- |
| In-lab | Task a | 10 points | 10 points | 10 points | 20 Points |
|  | Task b | 10 points | 10 points |  |  |
| Exercise (In-Lab) |  | 20 points | 10 Points |  |  |
|  |  | 40 | 30 | 10 | 20 |
| Total |  |  |  |  | 100 Points |

Marks Obtained:

|  |  | LR2 Code | LR5 Results | LR7 Viva (Performance) | LR9 Report |
| --- | --- | --- | --- | --- | --- |
| In-lab | Task a |  |  |  |  |
|  | Task b |  |  |  |  |
| Exercise (In-Lab) |  |  |  |  |  |
| Total |  |  |  |  |  |